# HW Trigger using 6843 Radar

# Hardware Requirements

- DCA1000EVM with modifications

- MMWAVEICBOOST with modifications

- IWR6843ISK (no modifications necessary)

- Microcontroller to send HW trigger pulses

# Software Requirements

- TI mmWave SDK 3.5.0.4

- TI Uniflash

- TI mmWave Studio 2.1.1.0

- Josiah’s single or dual radar GUI

  - Requires MATLAB

# Hardware Set Up

- DCA1000EVM

  - Remove R120

    - Suggested here

  - Connect to the MMWAVEICBOOST with 60 pin connector as shown on page 16 here

  - Connect to the PC over USB on the RADAR_FTDI/J1 connector and over Ethernet

  - Connect 5V/3A power

- MMWAVEICBOOST

  - Remove R346 and short 348

    - Suggested here and here

    - From Josiah’s E2E post:

      - Download xWR6843 EVM Schematic Drawing, Assembly Drawing, and Bill of Materials - SWRR164C.zip from here

      - On page 9 of PROC074B(001)_Sch.pdf (for rev B of the MMWAVEICBOOST), under "RNR FOR SYNC IN", 40PIN_SYNC_IN needs to be routed to RADAR_SYNC_IN

      - From that diagram, DCA_SYNC_IN is shorted via R346 to RADAR_SYNC_IN

      - Hence, remove R346 and place 0 ohm resistor over R348. Now 40PIN_SYNC_IN is routed to RADAR_SYNC_IN

  - Switch settings

    - From page 11 here, use the switch settings below on S1 for DCA1000 Mode

  - Connect to microcontroller (MCU)

    - From Josiah’s E2E post:

      - On page 8 of PROC074B(001)_Sch.pdf and page 19 of here, 40PIN_SYNC_IN is pin 9 of J5 (IMPORTANT)

        - See picture here for pin 9 of J5 input

      - Ground is pin 4 of J5 or pin 2 of J6

  - Connect to the PC over USB on XDS110_USB/J1 and attach IWR6843 as shown on page 16 here

  - Connect 5V/3A power

- IWR6843ISK

  - Switch settings

    - From step 1 here or page 45 here, use the switch settings below on S1

    - Connect to MMWAVEICBOOST as shown on page 16 here

# Software Set Up

- Flash the SDK demo to the MMWAVEICBOOST (See SDK 3.5.0.4 User Guide section 4.2)

  - Set the device to Flash Programming Mode by bridging SOP0 and SOP2 as shown in step 5 here

  - Power cycle the MMWAVEICBOOST

  - Once the device is properly connected to the PC, download the demo firmware using Unifash

    - Typically under the path: “C:\ti\mmwave_sdk_03_05_00_04\packages\ti\demo\xwr68xx\mmw”

  - Once the download is complete, set the device to Functional Mode by bridging only SOP0 and remove the bridge on SOP2

  - Power cycle the MMWAVEICBOOST

- Setup the DCA1000EVM on the proper IP address (See the DCA1000VEM Quick Start Guide)

  - Once the device is properly connected to the PC, open the start menu and search “View Network Connections”

  - Inside the “Network Connections” of Control Panel, right click on the Ethernet port of choice and select “Properties”

  - “Local Area Connection Properties” window will open. Right click on “Internet Protocol Version 4 (TCP/IPv4)”.

  - “Internet Protocol Version 4 (TCP/IPc4)” window will open. Set the IP address field to 192.168.33.30, or the desired IP address if different

    - See our other documentation on changing the IP address of the DCA1000EVM using the DCA1000 CLI Utility (necessary for a dual radar setup)

  - The Subnet mask field can remain the default 255.255.255.0

  - Press “OK” on all the windows and you can close “Network Connections”

- Open the Josiah’s single/dual radar GUI

    - “dual_radar_gui.mlapp.”

  - App Designer window will open to the app of your choice in the previous step

  - Press “Run” at the top of the page

  - The app will open and all the indicators will be red

    - Assuming you have installed mmWave Studio 2.1.1.0 to the typical location, it will open normally. Otherwise, it will ask you to find the installation location of mmWave Studio 2.1.1.0

  - Change the Start Freq (GHz) field to 60 GHz

  - Press “Connect Radar 1”

  - A window will appear asking to select a serial COM port. Select the COM port corresponding to the entry in device manager labeled “XDS110 Class Application/User UART”.

  - Press “Prepare DCA 1”

  - Enter the desired chirp parameters

  - Press “Configure Radar”

  - Press “Start” to start the capture

    - The DCA1000EVM will start waiting for data over LVDS

    - The MMWAVEICBOOST and IWR6843ISK will wait for HW trigger from MCU

      - If everything is working properly at this point, the D7 LED on the IWR6843ISK and the DS2 LED on the MMWAVEICBOOST will turn on

  - Start the MCU sending pulses

    - If everything is working properly the DATA_TRAIN_PRG LED on the DCA1000EVM will be flashing while the radar is triggered

  - Press “Stop” to stop the radar once you are done